NATIONAL SPACE HACKATHON 2026 – KEY FORMULAS, APIS & DATA

1. STATE VECTOR

S(t) = [x, y, z, vx, vy, vz]^T

2. ORBITAL MOTION EQUATION

d²r/dt² = - μ/|r|³ * r + a_J2

Constants:

μ = 398600.4418 km³/s²

RE = 6378.137 km

J2 = 1.08263 × 10^-3

3. J2 PERTURBATION

a_J2 = (3/2)*(J2*μ*RE² / |r|^5) * [

x(5z²/|r|² -1),

y(5z²/|r|² -1),

z(5z²/|r|² -3)

]

4. COLLISION CONDITION

|rsat - rdeb| < 0.100 km

5. ROCKET EQUATION

Δm = m_current (1 - e^(-|Δv|/(Isp*g0)))

Constants:

Isp = 300 s

g0 = 9.80665 m/s²

6. SATELLITE DATA

Dry mass = 500 kg

Fuel mass = 50 kg

Max Δv per burn = 15 m/s

Cooldown = 600 sec

7. APIs

Telemetry API:

POST /api/telemetry

Maneuver API:

POST /api/maneuver/schedule

Simulation API:

POST /api/simulate/step

Visualization API:

GET /api/visualization/snapshot

8. RULES

Prediction window = 24 hrs

Latency = 10 sec

Station drift tolerance = 10 km

9. GROUND STATIONS

ISTRAC Bengaluru (13.0333, 77.5167)

Svalbard (78.2297, 15.4077)

Goldstone (35.4266, -116.8900)

Punta Arenas (-53.15, -70.9167)

IIT Delhi (28.545, 77.1926)

McMurdo (-77.8463, 166.6682)

10. COORDINATES

ECI – global

RTN – maneuver frame

11. DEPLOYMENT

Docker required

Base image: ubuntu:22.04

Port: 8000